# Knowledge Check — Answer Key (Instructor)

FOR INSTRUCTOR USE ONLY.

## Question 1

What is the function of the PLC power supply, and what voltage does it typically output to the I/O modules?

Suggested answer: The PLC power supply converts incoming AC to low-voltage DC for the backplane and I/O. Field I/O is typically supplied 24 VDC (per cabinet design).

## Question 2

Describe the difference between a Normally Open (N/O) and Normally Closed (N/C) contact in ladder logic.

Suggested answer: N/O: open when de-energized, closes when true. N/C: closed when de-energized, opens when true.

## Question 3

A proximity sensor's LED is lit (indicating it is detecting a target), but the PLC input tag shows 0. What are two likely causes?

Suggested answer: Likely causes: wiring fault (open, loose terminal, fuse) or NPN/PNP/common mismatch so the input circuit does not complete.

## Question 4

What does OSSD stand for, and why do safety devices use two OSSD channels instead of one?

Suggested answer: OSSD = Output Signal Switching Device. Two independent channels so one fault cannot defeat the safety function.

## Question 5

You need to connect your laptop to an Omron PLC with IP address 192.168.5.20. What should you set your laptop's IP and subnet mask to?

Suggested answer: Same subnet as the PLC (e.g. 192.168.5.x with mask 255.255.255.0 for a 192.168.5.20 PLC).

## Question 6

In Studio 5000, what menu option do you use to browse the network and find controllers? What is the keyboard shortcut to go online?

Suggested answer: Communications → Who Active / Browse Chassis. Go online shortcut often Ctrl+D (confirm version).

## Question 7

A TON timer has a preset of 5000ms and an accumulator of 5000ms. Is the DN bit ON or OFF?

Suggested answer: DN is ON when accumulated time reaches/exceeds preset — at 5000/5000 ms, DN is ON.

## Question 8

What is a global variable, and how does it differ from a program-scoped tag?

Suggested answer: Global tags are controller-wide; program-scoped tags are limited to their program unless exposed.

## Question 9

List the three access levels on a typical HMI and describe what each level allows.

Suggested answer: Typical HMI levels: Operator, Maintenance/Technician, Administrator/Engineer (names vary).

## Question 10

At what escalation level should a technician address a servo drive STO (Safe Torque Off) fault?

Suggested answer: Level 2 — escalate servo/STO issues to Lead Technician / Noah Staudacher per policy.