Written Knowledge Check

Trainee copy · 10 questions · PLC Tabletop Training

Name: _________________________   Date: ______________   Score: _____ / 10

Answer in complete sentences unless your instructor directs otherwise.

Question 1

What is the function of the PLC power supply, and what voltage does it typically output to the I/O modules?

________________________________________________________________________

________________________________________________________________________

Question 2

Describe the difference between a Normally Open (N/O) and Normally Closed (N/C) contact in ladder logic.

________________________________________________________________________

________________________________________________________________________

Question 3

A proximity sensor's LED is lit (indicating it is detecting a target), but the PLC input tag shows 0. What are two likely causes?

________________________________________________________________________

________________________________________________________________________

Question 4

What does OSSD stand for, and why do safety devices use two OSSD channels instead of one?

________________________________________________________________________

________________________________________________________________________

Question 5

You need to connect your laptop to an Omron PLC with IP address 192.168.5.20. What should you set your laptop's IP and subnet mask to?

________________________________________________________________________

________________________________________________________________________

Question 6

In Studio 5000, what menu option do you use to browse the network and find controllers? What is the keyboard shortcut to go online?

________________________________________________________________________

________________________________________________________________________

Question 7

A TON timer has a preset of 5000ms and an accumulator of 5000ms. Is the DN bit ON or OFF?

________________________________________________________________________

________________________________________________________________________

Question 8

What is a global variable, and how does it differ from a program-scoped tag?

________________________________________________________________________

________________________________________________________________________

Question 9

List the three access levels on a typical HMI and describe what each level allows.

________________________________________________________________________

________________________________________________________________________

Question 10

At what escalation level should a technician address a servo drive STO (Safe Torque Off) fault?

________________________________________________________________________

________________________________________________________________________